SECTION 062022

Certified Wood

This 3-part specification is provided by Mithun as a design and procurement tool to assist in project development and documentation. It is offered for general informational and reference purposes only. By downloading or using this specification, the user acknowledges and agrees that Mithun assumes no responsibility or liability for the accuracy, completeness, suitability, or applicability of this document to any specific project or circumstance.

It is the sole responsibility of the user to review, verify, and adapt the contents of this specification to meet the unique requirements, codes, standards, and conditions of their particular project. Use of this document constitutes acceptance of these terms and a release of Mithun from any and all claims or liabilities arising from its use

EDIT NOTE: Any Related Sections in this specification must be edited to reference this section with the addition of the following language:

Part 2 - Materials – General
2.1. Procurement Criteria – wood material used in this section is to be Certified Wood, meeting the requirements of Section 062022.

EDIT NOTE: Revise this Section by deleting and inserting text to meet Project-specific requirements.

PART 1 - GENERAL

  - SUMMARY 
EDIT NOTE: Retain or delete wood certification subparagraphs that are not applicable to Project. 
If sourcing outside of the US and Canada, include PEFC. SFI is the PEFC-endorsed certification system in the US and Canada. 
Note that SFI is not included in the Climate-Smart Wood Group sourcing recommendations.
For non-certified salvaged or reclaimed wood, do not use USRW; use the 062020 Salvaged and Reclaimed Wood specification.

    - Section Includes:

      - Administrative and procedural requirements for use of certified forest products during performance of the Work. Certifications include:

        - Forest Stewardship Council (FSC) Certified Wood

        - Sustainable Foresty Initiative (SFI) Certified Wood

        - Urban Salvaged and Reclaimed Wood (USRW)

        - Programme for the Endorsement of Forest Certification (PEFC)

    - Related Sections:

### EDIT NOTE: Retain or delete subparagraphs below to identify Related Sections that require Salvaged or Reclaimed Wood sourcing.

      - Section 015639 “Temporary Tree & Plant Protection”

      - Section 018160 “Wood Supply Chain Disclosure Requirements”

      - Section 311000 "Site Clearing"

      - Section 033000 “Cast-in-Place Concrete”

      - Section 055010 “Landscape Metal Work”

      - Section 061000 "Rough Carpentry"

      - Section 061718 “Glue Laminated Timber”

      - Section 061719 “Cross Laminated Timber”

      - Section 062013 “Exterior Finish Carpentry”

      - Section 062023 “Interior Finish Carpentry”

      - Section 064023 “Interior Architectural Woodwork”

      - Section 064100 “Architectural Wood Casework’

      - Section 096400 “Wood Flooring”

## DEFINITIONS

### .

    - FSC Certified Wood: All wood products that are certified according to the rules of the Forest Stewardship Council’s FSC-STD-40-003 Chain of Custody Certification standard.

    - Certified Well-Managed Forests: Forests certified to be in compliance with the forest management standards established by the Forest Stewardship Council.

    - Chain of Custody (COC). All suppliers, including manufacturers, fabricators that control certified wood material from forest to installation in the Project and have received COC certification in compliance with the standards of the. For the purposes of this specification, the COC of a wood product refers to the COC as defined by the respective certification system(s) under which the product is certified.

    - SFI Certified Wood: All wood products that are certified according to the standards of the Sustainable Forestry Initiative.

    - PEFC Certified Wood: All wood products that are certified according to the standards of the Programme for the Endorsement of Forest Certification.

## PERFORMANCE REQUIREMENTS
EDIT NOTE: Retain or delete applications for which the preferred wood is to be used, coordinated with applicable Related Sections.

    - Provide Certified Wood for use in [interior and/or exterior] [architectural carpentry], [rough carpentry], [wood cladding], [wood flooring], [mass-timber fabrications] and [millwork] in accordance with Contract Drawings.

      - Certified Wood must meet all performance requirements outlined in Related Sections in this specification.

      - For all structural assemblies and components, Certified Wood must meet all structural requirements in accordance with the Contract Drawings and respective Related Sections.

## INFORMATIONAL SUBMITTALS
EDIT NOTE: Retain or delete subparagraphs below as necessary to document how the preferred wood meets project goals.

        - Submit “Wood Product Sourcing Transparency Form” as indicated in Section 018160 – “Wood Supply Chain Disclosure Requirements” for each source of Certified Wood source. EDIT NOTE: Certified Wood only, the Wood Product Transparency Form may not be necessary if the FSC COC certificates and documentation are deemed sufficient to meet project goals.

        - For all wood products designated in this specification as “FSC certified,” provide evidence of compliance with FSC standards as follows:

          - Demonstrate that products are FSC certified by providing vendor invoices. Invoices will contain the vendor’s Chain-of-Custody (COC) certificate number and identify each FSC certified product on a line-item basis. A “vendor” is defined as the company that furnishes wood products to project contractors and/or subcontractors for on-site installation.

          - FSC-certified products must be identified as such with their FSC claim (e.g., FSC 100%, FSC Mix Credit, FSC Mix XX%, FSC Recycled Credit, FSC Recycled XX%) by line item.

          - Wood products without submittal of acceptable documentation will be rejected.

        - For all wood products designated in this specification as “SFI certified,” provide vendor invoices demonstrating COC codes and the following claims. 
EDIT NOTE: Retain or delete the following SFI claims, as required by project goals. Note that SFI Certified Sourcing standard is not enforcedly different from legal wood in North America, which is covered in 2.1.A.1, and therefore is not an impactful claim for projects to pursue.

          - SFI 100% Certified Forest Content

          - SFI X% Certified Forest Content

          - SFI Credit, or 100% as calculated under the credit method

          - SFI X% Recycled Content

        - For all wood products designated in this specification as “USRW certified,” provide documentation required under USRW Chapter 3 Chain of Custody Requirements.

## QUALITY ASSURANCE

    - Shop Drawings:

      - Refer to required submittals in Related Sections [Architectural millwork, Architectural Finish Carpentry, Exterior Site Furnishings, etc].

    - All wood products designated as “FSC certified” in this specification shall be certified according to the rules of the Forest Stewardship Council (www.fscus.org).

    - All wood products designated as “SFI certified” in this specification shall be certified according to the rules of the 2022 Chain of Custody Standards of the Sustainable Forest Initiative (www.forests.org).

    - All wood products designated as “USRW certified” in this specification shall be certified according to the rules of the Urban Wood Sourcing Standards and Chain of Custody Standards of the USRW Certified Urban Wood system (www.usrwcertified.org).

# PART 2 - PRODUCTS

  - MATERIALS, GENERAL
EDIT NOTE: Retain or delete additional requirements as needed for wood applications. If the Related Sections for each application already address a requirement (typical), it should be deleted from this section. However, these provisions may be used to control other aspects of the Certified Wood across all related sections.

    - Provide [FSC-] [SFI-] [USRW-] Certified Wood. Certified Wood must meet the following requirements.

      - Legality. The wood must be legal in all jurisdictions through which the COC passes, considering all attributes, including harvest location, species, and extraction methods.

      - Species.   [As indicated in Related Sections and Contract Drawings.]

      - Size and Dimensions: as required by application as shown in Contract Drawings.

      - Relative moisture content: [Maximum [12] [15] percent at time of installation.] [As required by Related Sections]

      - Wood to be [air dried] or [kiln dried] to achieve relative moisture requirements.

      - Sourcing Radius: [Wood extraction to be within [100] miles of the Project site.] [Wood extraction to be within the maximum radius of Project sustainability requirements.]

      - Visual Grade. EDIT NOTE: Use for hardwoods as applicable to Project.  Minimum [FAS], [Selects] [No. 1 & 2 Common] per NHLA G-101 Standards. [As indicated in Related Sections].

    - Allowable substitute materials [permitted only when specified material supply is not sufficient for the Project]. EDIT NOTE: Identify acceptable alternatives for wood that meets Project sustainability goals in the case that the Certified Wood supply cannot meet the project requirements. Substitutes should typically have more stringent criteria (e.g. FSC as an allowable substitute for SFI) to avoid loopholes for avoiding desired criteria, but in cases where this is not possible, include the optional clause on circumstances when substitutions are allowed.

      - FSC-Certified Wood.

      - Salvaged Wood, procured under the criteria described in Section 062020. EDIT NOTE: Include Section 062020 if it is not already included in the Specifications.

      - Reclaimed Wood, procured under the criteria described in Section 062020.

      - PEFC-Certified Wood. EDIT NOTE: Listing PEFC is often necessary when projects are allowing wood to be sourced outside of the United States.

    - Certified Wood Suppliers

Acceptable suppliers of Certified Wood may be found at the following directory [directory

EDIT NOTE: Listing suppliers for Certified Wood is not always necessary, as the certification and COC criteria ensure that a wood product should meet the specification requirements. However, for certifications for products with a limited number of suppliers exist, e.g. FSC wood in cross-laminated timber, it can be helpful to list known suppliers.

      - Directory of FSC Forest Product Suppliers: https://search.fsc/en/

      - SFI Certificate Database: https://sfidatabase.org/

      - Directory of USRW wood suppliers: https://urbanwoodnetwork.org/directory/

EDIT NOTE: Add additional contacts that may be specific to the project.

      - Supplier Name

        - location(s) of operation

        - Phone number; email address

### EDIT NOTE: Specifications regarding fabrication of wood should be covered in the Related Sections pertaining to each specific application.

# PART 3 - EXECUTION

## EXAMINATION

    - Examine substrates, with Installer present, for compliance with requirements for installation tolerances and other conditions affecting performance of the Work.

    - Examine materials before installation. Reject materials that are wet, moisture damaged and/or mold damaged.

    - Proceed with installation only after unsatisfactory conditions have been corrected.

## PREPARATION

    - Wood to be prepared per Related Sections in this specification ahead of delivery to site or place of fabrication.

    - Clean wood if needed prior to installation.

## END OF SECTION